| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for BS-Controlled Direction Communication in 802.16.1a |  |
| Date Submitted | 2012-03-07 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to Letter Ballot #38 on P802.161a/D1 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for BS-Controlled Direction Communication in 802.16.1a

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

- This contribution proposes modifications for the existing specifications for BS-controlled HR-MS Direct Communications.

# References

- [1] IEEE P802.16.1a/D1, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Feb. 2012.

Proposed text changes for the IEEE P802.16.1a/D1

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify section 6.12.2.2.1.1 as indicated}

          - HR-MS Neighbor Discovery

HR-MS neighbor discovery is a key functionality to enable other 16n features such as path discovery and management, HR-MS direct communications (with or without presence of infrastructure), and HR-MS forwarding to network. HR-MS neighbor discovery procedures are specified for two scenarios: i) when HR-MSs associated with a common super-ordinate station (HR-BS/RS or a coordinating HR-MS) attempt to discovery each other and ii) when an out-of-coverage HR-MS attempts to discover an HR-MS in order to connect through it to network infrastructure.

To enable neighbor discovery among directly associated HR-MSs (use case 1), the super-ordinate station shall instruct these directly associated HR-MSs to transmit and receive predefined signals.

- 6.12.2.2.1.1.1 Neighbor discovery among associated HR-MSs

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast ranging signals so that other HR-MSs can try to receive and verify their neighbor relationship. The process can be described as follows:

- The serving HR-BS/HR-RS sends AAI-HR-RNG-CMD message to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

- Using AAI-HR-RNG-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for ranging signals.

- Each HR-MS that is scheduled to receive ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using AAI-HR-RNG-REP message.

The format of AAI-HR-RNG-CMD message is described in Error: Reference source not found. The HR-BS unicasts AAI-HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts AAI-HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the AAI-HR-RNG-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

{Remedy 2: Modify the sentence on line 6, page 113, just before Section 6.12.2.2.1.3.1 as indicated}

Synchronization mechanisms are specified for three two different use cases as follows.

{Remedy 3: Modify section 6.12.2.2.2.1 as indicated}

- 6.12.2.2.2.1 Frame structure and resource allocation

Resources for HR-MS Direct Communications and HR-MS Forwarding to Network shall be scheduled by the serving HR-BS/RS when one exists. Serving HR-BS/RS can schedule direct communication in an on-demand and dynamic manner, and can multiplex this with transmissions between HR-MS and HR-BS / HR-RS.

To optimize the signaling and switching cost and improve QoS provisioning to HR-MS direct communication, serving HR-BS / HR-RS can schedule resource for DC/FTN zone for multiplexing DC/FTN transmissions. An HR-MS DC / FTN Zone is an area of continuous OFDMA resources in time and logical subchannels or resource units. The size and location of DC/FTN zone is dynamically or semi-stationary determined by the serving HR-BS.

When an infrastructure node doesn’t exist, one of the HR-MS shall fulfill this coordinating role. It is understood that the coordinating HR-MS needs to take on some of the functionality of a HR-BS and may also require new functionality.

All resource scheduling shall be conveyed through MAP or DL control messages from serving HR-BS/RS or a coordinating HR-MS. In the case of HR-MS Forwarding to Network, the scheduling messages shall be forwarded by the forwarding HR-MS.

Random access channels may be used for bandwidth request. For case-1, bandwidth request are sent directly to the serving HR-BS /HR-RS. For case 2, bandwidth requests are forwarded by the forwarding HR-MS.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.