| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Technical Requirements document for MIH Security |  |
| Date Submitted | January 8, 2008 |  |
| Source(s) | Subir Das (Telcordia Technologies Inc.), Marc Meylemans (Intel Corporation), Yoshihiro Ohba (Toshiba), Lily Chen (NIST), Nadia Golmie (NIST), Maryna Komarova (ENST), Michael Williams (Nokia), Shubhranshu Sinha (Samsung) |  |
| Re: | IEEE 802.21 Session #24 in January 2008 |  |
| Abstract | This document describes the skeleton of an architectural and technical requirements document (TR) on MIH security. |  |
| Purpose | Security Study Group Discussion |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

IEEE 802.21 Security Study Group

Technical Report

Version 0.4

[Important note: the content of this draft Technical Report has not been approved by the 802.21 Security SG]

1	Introduction	5

1.1	Scope	5

1.1.1	Security signaling optimization during handovers	5

1.1.2	MIH Level security	6

1.2	References	6

1.3	Definitions and abbreviations	7

1.3.1	Definitions	7

1.3.2	Abbreviations	8

2	Security Signaling Optimization during Handovers	8

2.1	Use Cases	8

2.1.1	Applicable Handover Scenarios	8

2.1.2	General Requirements	9

2.1.3	General Assumptions	9

2.1.4	Use Case 1	10

2.1.4.1	Assumptions	10

2.1.4.2	Requirements	10

2.1.5	Use Case 2	10

2.1.5.1	Assumptions	11

2.1.5.2	Requirements	11

2.1.6	Use Case 3	11

2.1.6.1	Assumptions	12

2.1.6.2	Requirements	12

2.2	Potential Approaches	12

2.2.1	Potential Approaches Matrix	12

2.2.2	EAP Pre-Authentication	13

2.2.2.1	High-Level Architecture	15

2.2.2.2	Applicability of EAP Pre-Authentication	16

2.2.2.3	High-Level Requirements for EAP Pre-Authentication	16

2.2.3	Pro-active EAP Re-Authentication	16

2.2.3.1	Key Hierarchy	17

2.2.3.2	Protocol Interfaces	18

2.2.3.3	Applicability of Pro-active EAP Re-authentication	19

2.2.3.4	Requirements for Pro-active EAP Re-authentication	19

2.3	Example Call Flows	20

2.3.1	Example Call Flow for Pro-active EAP Re-Authentication	20

3	MIH Level Security	20

3.1	Use Cases	20

3.1.1	General Requirements	20

3.1.2	General Assumptions	23

3.1.3	Use Case 1	23

3.1.3.1	Assumptions	23

3.1.3.2	Requirements	24

3.1.4	Use Case 2	24

3.1.4.1	Assumptions	24

3.1.4.2	Requirements	25

3.1.5	Use Case 3	25

3.1.5.1	Assumptions	26

3.1.5.2	Requirements	26

3.1.6	Use Case 4	27

3.1.6.1	Assumptions	27

3.1.6.2	Requirements	28

3.1.7	Use Case 5	28

3.1.7.1	Assumptions	28

3.1.7.2	Requirements	29

3.2	Potential Approaches	29

3.2.1	General considerations	29

3.2.2	IETF recommendations	30

3.2.2.1	MIH Node discovery	30

3.2.2.2	MIH transport security	30

3.2.2.3	MIH-to-MIH authentication	30

4	Conclusions	30

5	Recommendations	30

Annex A: Security signaling during handovers – problem space	31

# Introduction

The IEEE 802.21 draft provides a media independent solution for handovers but does not address the security signaling required between heterogeneous networks. To perform a secure handover, the authorization and authentication between the MN and the target network needs to be addressed. However, security-related signaling can add significant delays to handover efforts. In many cases, service continuity can no longer be met, which affects time sensitive applications in particular, and ultimately impacts the user experience.

The other aspect is that the IEEE 802.21 draft does not address security at the MIH level. Because 802.21 provides services that affect network resources, cost and user experience, MIH level security will be an important factor to network providers that want to  deploy these MIH services in their network. MIH level security may comprise of the following aspects: mutual authentication of MIH peer nodes, access control to MIH services, MIH protocol integrity and replay protection, MIH protocol data confidentiality.

## Scope

The scope of this document is to describe usage scenarios, requirements and potential high-level approaches for the following two 802.21 Security Study Group topics:

  - Security signaling optimization during handovers

  - MIH-level security

### Security signaling optimization during handovers

This report considers mobile devices (MN) that have either a single radio or multiple radios to access 802-based networks (e.g. 802.11 and 802.16). Devices with only a single radio have certainly a more stringent requirement on security signalling delays during handovers, compared to devices that have multiple radios available that can work in an overlapping fashion during the handovers.

Each access network may have its own authentication scheme and key hierarchy. This report specifically addresses a security transition scheme between 802-based networks that are based on EAP authentication mechanisms.

The document outlines use cases whereby:

- A mobile device transitions between two 802-based access networks of different media types within the same administrative domain, e.g., 802.16 and 802.11

- A mobile device transitions between two 802-based access networks of the same media type and deployed in different administrative domains, e.g., 802.16

- A mobile device transitions between two 802-based access networks of different media types and deployed in different administrative domains, e.g., 802.16 and 802.11

The document also examines the requirements for and implications of:

- Network-specific aspects: changes required in access network or core network infrastructure for seamless transition between networks without comprising security

- Mobile client aspects: TBD (e.g. 802.16e/m, 802.11r)

- IETF HOKEY and other standard group aspects:  investigate if HOKEY covers all the targeted transition scenarios

### MIH Level security

With regards to MIH level security, the following security requirements are being considered and use cases are provided:

- Mutual authentication of MIH peer nodes

- Access control to MIH services

- MIH protocol integrity and replay protection

- MIH protocol data confidentiality

## References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

[1]	IEEE Draft Standard 802.21-D8.0: "Media Independent Handovers"

[2]	RFC 3748 – Extensible Authentication Protocol (EAP)

[3]	I-D: draft-ietf-hokey-preauth-ps-01.txt – EAP Pre-authentication Problem Statement

[4] 	IEEE Draft Standard 802.11r D8.0: ”Fast BSS Transition”

[5]	21-07-0127-00-0000-Hokey-802.21.ppt, ”Handover security in a heterogeneous Access Environment IETF HOKEY-IEEE 802.21 Integration”

[6] 	21-07-0122-04-0000-Security_proposal.ppt, “Security Optimization During Handovers: 802.21 SG Proposal”.

[7] I-D: draft-ietf-hokey-emsk-hierarchy-01.txt

[8] I-D: draft-ietf-hokey-erx-05.txt

[9]  RFC 1136 – Administrative Domains and Routing Domains: A model for routing in the Internet

[10] Draft-melia-mipshop-mstp-solution-01: “Mobility Services Transport Protocol Design”

[11] Draft-ietf-mipshop-mis-ps-05: “Mobility Services Transport: Problem Statement”

[12] Draft-rahman-mipshop-mih-transport-03: Transport of Media Independent Handover Messages Over IP

[13] Draft-hepworth-mipshop-mih-design-considerations-01: Design Considerations for the Common MIH Protocol Functions

[14] IEEE Draft Amendment to Standard P802.11u/D1.02: Interworking with External Networks

[15] IEEE Draft Std 802.16g-2007 September 2007

[16] RFC 4907:  Architectural Implications of Link Indications, June 2007

## Definitions and abbreviations

### Definitions

Administrative domain: A collection of end systems, intermediate systems, and sub-networks operated by a single organization or administrative authority. The components which make up the domain are assumed to interoperate with a significant degree of mutual trust among themselves, but interoperate with other ‘Administrative Domains’ in a mutually suspicious manner. Administrative Domains can be organized into a loose hierarchy that reflects the availability and authoritativeness of authentication and authorization information.  This hierarchy does not imply administrative containment, nor does it imply a strict tree topology (definition based on [9])

Authenticator: EAP Authenticator [2]

Candidate Authenticator: The authenticator on a candidate Point of Attachment (PoA) with respect to the mobile node.

EAP Pre-authentication:  The utilization of EAP to pre-establish EAP keying material on an authenticator prior to attaching the peer to the access network managed by that authenticator.

Peer: EAP Peer [2]

Serving Authenticator: The authenticator on the serving PoA

Target Authenticator: The authenticator on the target PoA

### Abbreviations

In addition to the abbreviations that are defined in the 802.21 draft specification, the following abbreviations are defined:

.

BS		Base station in 802.16 networks

CA		Candidate Authenticator

CS		Command service of the media independent handover services

ERS	EAP Re-authentication Server

ES		Event service of the media independent handover services

IAB	Internet architecture board

IS		Information service of the media independent handover services

FQDN 	Fully qualified domain name

MIH	Media independent handover services

MIHF	Media independent handover services function

MN	Mobile node

MoS	Mobility services (as defined in [10]) - includes the MIH IS, CS and ES)

MoSh	Mobility service in home network

MoS3	Mobility service in third party network

MoSv	Mobility service in visited network

MS		Mobile station in 802.16 networks

MSTP	Mobility services transport protocol

NWDS	Network detection and selection

PFS	Perfect forward secrecy

PoA	Point of attachment

PoS	Point of service

SA		Serving Authenticator

TA		Target Authenticator

# Security Signaling Optimization during Handovers

## Use Cases

### Applicable Handover Scenarios

Figure 1 below depicts which handover scenarios are addressed in this report.

Intra-technology

Intra-domain

Inter-domain

Inter-technology

EAP to non-EAP

Yes

No (for

future study)

Inter-technology

EAP to EAP

No

Yes

Yes

No (for

future study)

Figure 1: Applicable handover scenarios

### General Requirements

The following requirements are common for all use cases outlined in Section 2.1:

R0.1 The subscriber possesses a MN which gives access to 802 access networks. For example; the MN can have either one single radio or two distinct radios that provide both 802.11 and 802.16 network access.

R0.2 The MN shall support functionality to perform a seamless transition between the access networks to which it is allowed for access.

R0.3 The network shall support the functionality to allow MN to perform seamless transition from one access network to another.

R0.4 Transition between networks shall be automatic and shall not require the   manual intervention of the user. The Network and MN should work in tandem to provide the most optimal handover experience.

R0.5 It shall conduct an authentication during handover to a different media. That is, a transition to a different media shall be authorized based on an authentication.

R0.6 The handover procedure shall establish new session keys to be used for lower layer protection.

R0.7 The security protection after handover shall comply with the security level required by the media which the MN transitions to.

R0.8 The resource consumption (including network traffic and power consumption) of the authentication and key establishment in the handover should be minimized with respect to a full EAP authentication.

R0.9 The delay caused by the authentication and key establishment in the handover should be minimized.

### General Assumptions

The following assumptions are common for all use cases outlined in Section 2.1:

A0.1 EAP ([2]) is used as the access authentication protocol for each of the media types. The EAP methods provide mutual authentication and required key material.

A0.2 A MN is authenticated with the serving authenticator through an EAP method.

A0.3 MN shall be able to discover candidate authenticators.

A0.4 Furthermore, HOKEY key hierarchy ([5] and [7]) may be supported by the employed EAP methods.

### Use Case 1

A mobile device transitions between two 802-based networks of different media types (e.g., 802.11 and 802.16) within the same administrative domain.

Figure 2: Handover scenario between 802.11 and 802.16 networks (single administrative domain)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

### Use Case 2

A mobile device transitions between two networks of different media types (e.g., 802.16 and 802.11) and deployed in different administrative domains.

Figure 3: Handover scenario between 802.11 and 802.16 networks (different administrative domains)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

### Use Case 3

A mobile device transitions between two networks with the same media types (e.g., 802.16) and deployed in different administrative domains.

Figure 4: Handover scenario between two 802.16 networks (different administrative domains)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

## Potential Approaches

### Potential Approaches Matrix

Intra-technology

Intra-domain

Inter-domain

Inter-technology

EAP to non-EAP

As defined in the technology specific key hierarchy, like 802.11r or 3GPP

Key hierarchy based solution

May need to establish mapping between different authentication mechanisms

(For future study)

EAP Pre-authentication based solution

Inter-technology

EAP to EAP

May need to establish mapping between different authentication mechanisms

(For future study)

Figure 5: Potential approaches matrix

### EAP Pre-Authentication

The following usage scenario is considered for security signaling optimization during handover:

- The peer is connected to the serving network.

- The peer or the serving network discovers one or more candidate network(s) in its neighborhood. The candidate network(s) and the serving network may have different link-layer technologies or may be in different IP subnets or administrative domains.

- The peer or the serving network chooses one or more candidate authenticator(s) among the discovered candidate network(s) and initiates EAP pre-authentication to be performed between the peer and authenticator on the candidate network(s).

- A successful EAP pre-authentication for candidate authenticator(s) will generate EAP keying material that is delivered from the EAP server to the authenticator(s) of candidate network(s).

- After EAP pre-authentication, the peer or the serving network chooses one target authenticator in one of the candidate network(s), and a handover procedure will take place to switch from the serving authenticator to the target authenticator.

- In the handover procedure, the EAP keying material generated during the EAP pre-authentication process will be used for establishing a session key (or session keys) to be used for protecting link-layer frames exchanged between the peer and the authenticator of the target access network.

There are two modes of operations to establish EAP pre-authentication depending on whether the pre-authentication signaling is transparent to the serving authenticator or not. Direct pre-authentication is the mode in which the pre-authentication signaling is transparent to the serving authenticator (see Fig.6). Indirect pre-authentication is the mode in which the authenticator participates in the pre-authentication signaling (see Fig.7).

Serving PoA

M

I

H

MN

R1

M

I

H

P

oS

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-CA Signaling

EAP over L2 or

higher layers (HL)

EAP over AAA

R2

Figure 6: Direct pre-authentication model

Serving PoA

M

I

H

MN

R1

R2

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-SA Signaling

EAP over L2/HL

EAP over AAA

SA-CA Signaling

EAP over L2/HL

SA  Serving Access Network

CA  Candidate Access Network

MIH PoS

Figure 7: Indirect pre-authentication model

#### High-Level Architecture

Figure 8: Reference model

The following reference points defined in 802.21 are reused to support EAP pre-authentication, with additional functionalities as described below.

R1: MN-SA signaling messages are exchanged over this reference point.

R2: MN-CA signaling messages are exchanged over this reference point.

R5: SA-CA signaling messages are exchanged over this reference point.

#### Applicability of EAP Pre-Authentication

EAP pre-authentication has general applicability to various deployment scenarios in which proactive signaling can take effect.  In other words, applicability of EAP pre-authentication is limited to the scenarios where candidate authenticators can be easily discovered and an accurate prediction of movement can be easily made.

The effectiveness of EAP pre-authentication is more significant for intra-technology and inter-subnet handover scenarios, and particular inter-technology handover scenarios where simultaneous use of multiple technologies is a major concern or where there is not sufficient radio-coverage overlap among different technologies.

#### High-Level Requirements for EAP Pre-Authentication

Requirement 1: MIH PoS shall support the functionalities of authenticator for EAP pre-authentication

Requirement 2: MN shall support the functionality of peer for EAP pre-authentication

Requirement 3: An authenticator discovery mechanism shall be defined. The authenticator discovery mechanism must provide a mapping between a link-layer address and an IP address of an authenticator.

Requirement 4: A context binding mechanism shall be defined so that a link-layer specific security context is bound to the EAP keying material generated as a result of EAP pre-authentication. The link-layer specific security context includes link-layer addresses of a peer and an authenticator.

Requirement 5: Higher-layer transport shall be supported for carrying EAP pre-authentication messages between MN and CA, between MN and SA and between SA and CA

Requirement 6: Link-layer transport shall be supported for carrying EAP pre-authentication messages between MN and SA

Requirement 7: The EAP pre-authentication process shall define a ‘lifetime’ parameter (pre-authentication validity time-out)

### Pro-active EAP Re-Authentication

The following usage scenario is considered for security signaling optimization during handover:

- The MN is connected to the serving network.

- The MN or the serving network discovers one or more candidate network(s) in its neighborhood. The candidate network(s) and the serving network have different link-layer technologies.

- The MN or the serving network chooses one or more candidate authenticator(s) among the discovered candidate network(s) and initiates EAP re-authentication ([8]). It is to be performed between the MN (as the peer) and an authenticator on the candidate network(s).

- Upon a successful EAP re-authentication, the authenticator will receive an rMSK that is delivered from the EAP server (or a local server).

- After EAP re-authentication, a handover procedure will take place to switch from the serving authenticator to the target authenticator.

- In the handover procedure, rMSK may be used for establishing intra-technology handover keys for different PoAs or session keys to be used for protecting link-layer frames exchanged between the MN and a PoA, depending on media specific handover.

The following items are for further study in 802.21 Security Study Group:

- Due to the optimization in security signaling by using EAP re-authentication, MN may or may not need to re-authenticate with multiple candidate authenticators.

- For the same reason MN may or may not use early initiated re-authentication with the target authenticator.

- Similar to EAP pre-authentication, MN either directly authenticates to the target or candidate authenticator or indirectly through the serving authenticator.

#### Key Hierarchy

The key hierarchy is derived using EMSK by the EAP authentication server or using DSRK by a local authentication server.  The re-authentication root keys (rRKs) are derived from EMSK or DSRK.

A re-authentication integrity key (rIK) is derived from rRK to provide integrity protection in the re-authentication between a peer (MN) and an EAP re-authentication server (ERS). MN and ERS communicate through the target authenticator, where ERS may be the EAP server / home server or a local server which has previously obtained an rRK.

Upon a successful re-authentication, an rMSK is delivered to the target authenticator (TA).  Fig. 2 illustrates the HOKEY key hierarchy.

rRK-1

rRK-2

rMSK-2

rMSK-1

EMSK or DSRK

rIK-1

rIK-2

Figure 9: HOKEY key hierarchy

#### Protocol Interfaces

The protocol interface is described by Fig.10

MN

SA

TA

ERS

1. Current connection

2. Re-authentication

3.rMSK

4. New connection

Figure 10: Protocol interfaces

#### Applicability of Pro-active EAP Re-authentication

EAP re-authentication has general applicability to handovers within the same domain between different media types which use EAP as an access authentication. Similar to EAP pre-authentication, it applies to the scenarios where candidate authenticators can be easily discovered and an accurate prediction of movement can be easily made.

The efficiency of EAP re-authentication applies to the scenario where an administration provides service to different media types. Re-authentication for optimized security signaling enables handovers with minimum delay and minimized communication with the home server so that the end users can maintain their quality of service level without sacrificing the scarce resources (such as battery power and bandwidth).

#### Requirements for Pro-active EAP Re-authentication

Requirement 1: MIH PoS shall support the functionalities of authenticator for EAP re-authentication.

Requirement 2: MN shall support the functionality of peer for EAP re-authentication.

Requirement 3: An authenticator discovery mechanism shall be defined. The authenticator discovery mechanism must provide a mapping between a link-layer address and an IP address of an authenticator.

Requirement 4: A context binding mechanism shall be defined so that a link-layer specific security context is bound to the EAP keying material generated as a result of EAP re-authentication. The link-layer specific security context includes link-layer addresses of a peer and an authenticator.

Requirement 5: Higher-layer transport shall be supported for carrying EAP re-authentication messages between MN and CA/TA and between CA/TA and ERS.

Requirement 6: Link-layer transport shall be supported for carrying EAP re-authentication messages between MN and CA/TA.

Requirement 7: There shall be a trust relationship between each CA/TA and ERS, which may be established via mutual authentication.

Requirement 8: If an ERS is a local authentication server, then there shall be a trust relationship between the ERS and home EAP server, which may be established via mutual authentication.

Requirement 9: There shall be a protected channel for confidentiality and integrity between each CA/TA and the ERS for the rMSK delivery.

## Example Call Flows

### Example Call Flow for Pro-active EAP Re-Authentication

MN

TA

ERS

[EAP Request/identity]*

[EAP Initiate /Re-auth Start]

rMSK, EAP Finish/Re-auth

EAP Initiate/Re-auth

EAP Initiate/Re-auth

EAP Finish/Re-auth

Figure 11: EAP Re-authentication message flow

The first two messages are optional. It is also possible that the re-authentication is initiated by MN (peer) since it may indicate its re-authentication capability.

# MIH Level Security

## Use Cases

### General Requirements

Based on the scoping work done in other groups (above) and based analysis of the MIH protocol itself, the following is the summary of aspects that need use cases to be provided:

  - Security of MIHF discovery and MIHF capability discovery:

    - There are two kinds of transport mechanisms: the lower layer transport (L2) and the MSTP layer transport.

    - MIHF discovery: over media-specific L2 or higher layer mechanism

    - MIHF capability discovery: either over MIH or over media-specific broadcast messages

  - Mutual authentication of MIH peer nodes

    - No authentication is defined by MIH [1] during the process of MIHF discovery and MIH capability discovery.

    - In the CS, the MN authorizes the CS MIHF to issue commands, so the MN may want to authenticate the CS.  The CS PoS may be willing to direct unknown MNs, or may require authentication.

    - In the IS, the IS MIHF may want to control access to the information based on authorization, so the IS MIHF may require authentication of the MN. The MN MIHF may or may not care if the IS MIHF is known.

    - In the ES, the subscriber to the events may or may not care if the generating MIHF is authentic. It is also possible the generator may want to allow subscription for events to authorized remote MIHF only.

  - MIH based access control

    - After authentication different MN MIHF may be authorized to access all or only limited services. This may help with DoS issues.

    - Per-MN management of access rights may be needed, with additional considerations:

      - MN may not be known in advance (if belonging to a different administrative domain.)

      - MN may not disclose its identity to a visited network.

    - Role-based management of access rights may be implemented. The role may be based on some aspect of the MN’s state (unauthenticated/authenticated) or its subscription (home/visiting).

    - In some implementations the MN MIHF should be able to select the most well known IS MIHF among all available.

  - Security of MIH Protocol

    - MIH protocol integrity and replay protection

    - MIH protocol confidentiality.

Evaluating this list, it is clear the same use cases or scenarios posed in [10] are useful for considering the MIH services and protocol security. There are some additional scenarios introduced due to the possibilities from [14] and [15]. The additional scenarios have to do with using the IS in unauthenticated state. When deriving requirements from the use cases, the security requirements or aspects described in [10] [11] [14] [15] and [16] are also considered against the scenario.

Apart from use cases derived from network architecture, there are some considerations native to the MIH services themselves that can generate use cases and related security requirements.

  - Information Service

    - Discovery may originate within or from outside administrative domain boundaries.

    - In access networks such as 802.11 and 802.16 an MN should be able to obtain IEEE 802.21 related information elements before the MN is authenticated with the PoA, to enable best network detection and selection (NWDS.)

    - In order to protect the user from receiving wrong information, it should be possible for the IS to be authenticated to the MN.

    - It should be possible for the IS to define different sets of information available for MNs in authenticated and un-authenticated states.

  - Event Service and Command Service

    - Mutual authentication between the NN MIHF and the MN MIHF should be used.

    - Secure channel establishment.

    - Providing confidentiality, integrity protection and message origin authentication.

Requirements

R0.1 It shall be possible to discover an MIH PoS while attached to that PoS’s home network.

R0.2 It may be possible to discover an MIH PoS when visiting a roaming partner’s network.

R0.3 It may be possible to discover an MIH PoS while attached to a 3rd party network.

R0.4 It may be possible to exchange IS messages between the MN and PoS before the MN is authenticated to the network, or authenticated to the IS MIHF.

R0.5 It may be possible for the MIHF IS to provide different results to information requests based on the identity or authorization of the requesting MN MIHF.

R0.6 It shall be possible to enforce mutual authentication between the CS MIHF and the MN MIHF.

R0.7 It may be possible for a CS MIHF to provide service to an unauthenticated MN MIHF.

R0.8 It shall be possible to enforce mutual authentication between the ES MIHF and the MN MIHF.

R0.9 It may be possible for the MIHF generating the events to provide service to an unauthenticated MN MIHF.

### General Assumptions

The current 802.21 draft does not specify the location of the MoS. The IS, ES and CS may be located in the serving, candidate or home network or it can even be managed by a third party authority. The document [10] describes four scenarios for communication between the MN and Mobility Services (MoS).

### Use Case 1

The MN and the PoS are located in the MN’s home domain, and the MN is authenticated and authorized on the network. In this situation the MN trusts the PoS and it may use either L2 technology specific mechanisms or DHCP/DNS for service discovery.

It is not always possible to establish link-layer security between two MIHF (for example, the MN and the PoS are located on different links). Also, in some home network implementations, link layer access is left open, but access is only available by establishing security at a higher layer. In this case L3 or above must provide security. Deployment options for secure network access could include link encryption, IPSec tunnel to a VPN gateway, SSL VPN and others. In these cases the MIH services are unaware which is being used to provide secure access to the home network.

Even if the MN and the PoS are located in the home domain some issues must be considered. The home domain may span several networks. Because the network architecture of the MIH services is not specified, the PoS for IS, ES and CS may be located in different networks belonging to the home domain. Since they may need to communicate with each other, there will need to be security in place between them.

Using the requirements listed in the problem space history, and the following assumptions, we can generate requirements from this scenario.

#### Assumptions

In this use case, the following observations are made to generate requirements:

MN:

A1.1 The MN has authenticated and been authorized for network access.

A1.2 If there is link security in use on the network, it is established between the MN and point of attachment (PoA.)

A1.3 The MIHF in the MN relies on existing link layer security, network layer security, MIH transport security and for confidentiality and integrity protection. PFS is not needed.

A1.4 The MN does not need DoS protection or replay protection from home domain MIHFs.

A1.5 The MN trusts the validity of the services based on the use of standard MIH services.

NN:

A1.a The MIHF for IS, CS and ES have transport-based (MSTP) security associations established between each other within the home domain if needed, as determined by the security implementation within the home domain. Data integrity and confidentiality are in place for messages exchanged between these PoSs. PFS is not needed.

A1.b Replay protection and DoS prevention is not needed between PoS in the home domain.

A1.c The MIHFs in the network rely on the existing access control mechanisms of link, network and transport security for DoS protection and replay protection from MN attacks. There is no additional protection against trusted but malicious MN MIHF.

#### Requirements

R1.1 When the MIHF is in a network node (NN) in the home domain, there shall be a service that indicates to the MIHF that is in the home domain and the other NN MIHFs are also in the home domain.

R1.2 When the MN MIHF is attached to the home domain, there shall be a service that indicates to the MIHF that it is being served by home MIHFs through the home domain.

R1.3 It may be possible for the MIHF in a NN PoS to require and perform authentication and authorization of the home MN MIHF. There shall be a service specified for the MN to receive or be denied authorization from the PoS.

R1.4 Authorization may be required in the case of the home MN MIHF performing capability discovery with a home NN MIHF.

### Use Case 2

The MN MIHF is located in a visited domain, communicating to the visited domain’s NN MIHFs. Link-layer security is established between the MN and its PoA, based on roaming agreements. After performing mutual authentication between the MN and the visited network’s AAA server to establish link layer security and network authorization, the MN trusts network entities to send mobility related information and commands and the network authorizes the MN to access mobility services according to the authorization profile associated with the MN. Within the visited network, security is established between the NN MIHFs.

#### Assumptions

The following observations are made to be used to generate requirements:

MN:

A2.1 The MN has authenticated and been authorized for network access.

A2.2 If there is link security in use on the network, it is established between the MN and point of attachment (PoA.)

A2.3 The MIHF in the MN relies on existing link layer security, network layer security and MIH transport security for confidentiality and integrity protection. PFS is not needed.

A2.4 The MN does not need DoS protection or replay protection from visited domain MIHFs.

A2.5 The MN trusts the validity of the services based on the use of standard MIH services.

NN:

A2.a The MIHF for IS, CS and ES have transport-based security associations established between each other within the visited domain if needed, as determined by the security implementation within the visited domain. Data integrity and confidentiality are in place for messages exchanged between these PoSs. PFS is not needed.

A2.b Replay protection and DoS prevention is not needed between PoS in the visited domain.

A2.c The MIHFs in the network rely on the existing access control mechanisms of link, network and transport security for DoS protection and replay protection from MN attacks. There is no additional protection against trusted but malicious MN MIHF.

#### Requirements

R2.1 When the MN MIHF is in a visited domain, there shall be a service that indicates to the visited NN MIHFs that the communicating MN MIHF is visiting.

R2.2 When the MN MIHF is attached to a visited domain, there shall be a service that indicates to the MN MIHF that it is being served by visited MIHFs through the visited domain.

R2.3 It shall be possible for the MIHF in a visited NN PoS to require and perform authentication and authorization of the MN MIHF while it is visiting. There shall be a service specified for the MN to receive or be denied authorization from the PoS.

R2.4 Authorization may be required in the case of the visiting MN MIHF performing capability discovery with a visited NN MIHF.

### Use Case 3

In this scenario the MN is located in the visited network and the PoS is located in the home network. Link-layer security is established between the MN and its PoA, based on roaming agreements. The visited network may not provide the needed MIH PoS, or the roaming agreement may specify that visiting MNs are to use their home PoS for some services. Also, the MN may be configured to use only its home PoS when visiting.

The MN must be pre-configured with identity of the home NN MIHF. The MN may be pre-configured with the IP address of the home NN MIHF as well. If it is not preconfigured with the IP address, the MN should be able to discover the home NN MIHF in a secure manner. When the MN communicates with the IS only server-to client authentication is required, while mutual authentication should be performed between the MN and ES or CS PoS before MIHF registration.

#### Assumptions

MN:

A3.1 The MN has authenticated and been authorized for network access.

A3.2 If there is link security in use on the network, it is established between the MN and point of attachment (PoA.)

A3.3 The MIHF in the MN relies on existing link layer security, network layer security and MIH transport security for confidentiality and integrity protection. PFS is not needed.

A3.4 The MN does not need DoS protection or replay protection from home domain MIHFs, even while visiting another network.

A3.5 The MN trusts the validity of the services based on the use of standard MIH services.

NN:

A3.a The MIHF for IS, CS and ES have transport-based security associations established between each other within the home domain if needed, as determined by the security implementation within the visited domain. Data integrity and confidentiality are in place for messages exchanged between these PoSs. PFS is not needed.

A3.b Replay protection and DoS prevention is not needed between PoS in the visited domain.

A3.c There is secure communication between the home and visited domains that carries the MIH traffic between the home PoS and the away MN. The MIHFs in the network rely on the visited domain’s existing access control mechanisms of link, network and transport security for DoS protection and replay protection from MN attacks. There is no additional protection against trusted but malicious MN MIHF.

#### Requirements

R3.1 When the MN MIHF is in a visited domain, there shall be a service that indicates to the home NN MIHFs that the communicating MN MIHF is visiting a roaming partner.

R3.2 When the MN MIHF is attached to a visited domain, there shall be a service that indicates to the MN MIHF that it is being served by its home NN MIHFs through the visited domain.

R3.3 It may be possible for the MIHF in a home NN IS PoS to require and perform authentication and authorization of the MN MIHF while it is away. In this case there shall be a service specified for the away MN to receive or be denied authorization from the home PoS.

R3.4 It shall be possible for the away MN MIHF to authenticate the home IS MIHF.

R3.5 Authorization may be required in the case of the away MN MIHF performing capability discovery with a home NN MIHF.

R3.6 It shall be possible for the away MN MIHF to mutually authenticate the home NN ES MIHF or home NN CS MIHF.

### Use Case 4

The MN is located in either the visited or in the home network, and the PoS is located in a 3rd party network. For example, the MN MIHF is subscribed for MIH from a provider of such services that is not related to the home network operator or the visited network operator. A provider might be an enterprise intranet MIH PoS, or a public company offering MIH by subscription.

MSTP or other higher layer security should be implemented in this case to create a secure path between the MN and the 3rd Party PoS. In order to establish trust between the MN and the 3rd party NN MIHF, serving IS MIHF should authenticate itself to the MN MIHF. The MN may or may not need to authenticate to the IS MIHF. To avoid communication between the MIHF and unauthorized MNs, mutual authentication should be performed between the MN MIHF and the MIHF providing ES and CS.

#### Assumptions

MN:

A4.1 The MN has authenticated and been authorized for network access.

A4.2 If there is link security in use on the network, it is established between the MN and point of attachment (PoA.)

A4.3 The MIHF in the MN cannot rely on existing link layer security or network layer security that is used to attach and access the network for confidentiality nor for integrity protection. PFS is not needed. It must assume an insecure path to the 3rd party PoS.

A4.4 The MN may need DoS protection and replay protection from the insecure path to the 3rd party provider PoS..

A4.5 The MN trusts the validity of the services based on the use of standard MIH services.

NN:

A4.a The MIHF for IS, CS and ES have transport-based security associations established between each other within the 3rd party domain if needed, as determined by the security implementation within the 3rd party domain. Data integrity and confidentiality are in place for messages exchanged between these PoSs. PFS is not needed.

A4.b Replay protection and DoS prevention is not needed between PoSs in the 3rd party domain.

A4.c There is no secure communication path between the domain of the MN’s PoA to carry the MIH traffic from the MN to the  3rd party PoS and back. The MIHFs in the 3rd party network cannot rely on the PoA’s domain for network access control mechanisms or link, network and transport security. DoS protection and replay protection is needed from path based attacks. There may need to be protection against trusted but malicious MN MIHF.

#### Requirements

R4.1 When the MN MIHF wants to use a 3rd party MIHF, it shall be possible to create a secure transport connection between the MN and the PoS. There must be an indication to each MIHF that the connection between them is secured.

R4.2 It shall be possible for the away MN MIHF to mutually authenticate with the 3rd party NN MIHF, regardless of the service type.

R4.3 Authorization may be required in the case of the away MN MIHF performing capability discovery with a 3rd party NN MIHF.

### Use Case 5

The MN is accessing the IS while the MN is not unauthenticated to the network.  The network authorizes unauthenticated use of MIH IS for expediting NWDS only. The information is expected to be hints and considered ‘better than nothing.’ The network is not sure of the MN’s identity, so does not know if the MN is part of the home network or to be treated as roaming / visiting, or as third party. The MN may detect broadcast information or use other methods to attempt to identify or even authenticate the network PoA and PoS. However the MN may not be certain if the network is home, a roaming partner, or a third party network.

#### Assumptions

MN:

A5.1 The MN has not been authenticated for network access.

A5.2 There is no link security established between the MN and its PoA.

A5.3 The MN MIHF must use IS PoS access methods provided within the link technology to address and exchange messages.

A5.4 There are no access methods available at this time for using ES or CS even if they are advertised in the link technology beacon.

A5.5 The MN may need DoS protection and replay protection from the insecure path to the PoS.

A5.6 The MN uses the unsecured IS service ‘as is’ without assuming the provider is trusted.

NN:

A5.a The IS MIHF may be reached through the PoA without authenticating to the network first.

A5.b Replay protection is not needed between PoA and the IS PoS.

A5.c There is a secure connection between the PoA and the IS PoS so confidentiality and integrity of messages between them is provided.

A5.4 DoS protection  is needed.

#### Requirements

R5.1 It may be possible to have message originator authentication between the MN MIHF and the IS MIHF even when the MN is not authenticated to the network.

R5.2 It may be possible for the MN MIHF and IS MIHF to use message integrity protection even though the MN is not authenticated to the network.

R5.3  It may be possible for the MIHF IS to receive an indication that an incoming IS message is from an unauthenticated MN MIHF.

## Potential Approaches

### General considerations

Regarding security of MIH Protocol the following should be taken into consideration to aid in acceptance and increase ease of deployment of the solutions:

- Re-using existing transport protocol security (as specified in MSTP)

- Re-using existing solutions for authentication, authorization, confidentiality and message integrity (also specified in MSTP)

### IETF recommendations

#### MIH Node discovery

### The document [10] provides solutions for MIHF discovery. If DHCP is used for node discovery, it is recommended to use DHCP authentication option (RFC3118). This solution provides mechanisms both for node authentication and message authentication.

### If DNS is used, it is recommended to use DNSSEC (RFC 4033).

#### MIH transport security

### In case when a reliable transport protocol such as TCP is used for connection between two MIHF peers, TLS (RFC 4366) should be used for data integrity and confidentiality.

### In case where unreliable transport protocol is used for connection between two MIHF peers, DTLS (RFC 4347) may be used.

### Alternatively, for generic IP level security, IPSec (RFC 2401) may be used if nether transport level security for a specific transport is available nor server only authentication is required.

#### MIH-to-MIH authentication

### Using FQDN and NAI as MIHFID is applicable in all mentioned scenarios. IP address may be used as MIHFID in case where the MN and the MoS are located in the same network (Use Case 1 and Use Case 2) or the IP address of MoS is preconfigured on the MN.

# Conclusions

# Recommendations

This section shall contain a list of requirements agreed to be potentially transferred to specifications.

# Annex A: Security signaling during handovers – problem space

To be added

Annex B: MIH level security – problem space

In the MIH services framework, the protocol carrying mobility services flows between two MIH functions (MIHF). An MIHF providing one of the services, event service (ES), information service (IS), or command service (CS) in the network is a point of service (PoS.) The MIH protocol can travel between a mobile node (MN) and a PoS, or between two PoS.  Out of scope for this discussion is the possibility of two MN exchanging the MIH protocol.

In the current IEEE P802.21 draft [1] the general problem space of security for the MIH protocol and services is unspecified. In section 1.3, the document says security mechanisms are out of scope. There are a few mentions of security in the draft [1] however:

- In 3.40, seamless handover is defined as having ‘no degradation in … security… ‘

- In section 5.3.4 (Information Service): “this standard enables both L2 and L3 transport options for this information and expects the transport to provide security for this information.”

- Also 5.3.4: “MIIS either relies on existing access media specific transports and security mechanisms or L3 transport and L3 security mechanisms to provide access to the information.”

- In the MIIS IE space, there are several IEs which refer to the security in use within a network, such as the Network Security IE and related query, with values such as “strong-cipher” and “open.”

In [10] the IETF MIPSHOP Working Group describes four scenarios for securing communication between the mobile node (MN) and MIH Mobility Services (MoS). The problem space is organized in two dimensions: if the mobile and the MoS are in home (MoSh), visited (MoSv) or third party (MoS3) networks; and if the transport is reliable or unreliable. Some points to note:

- Standard authentication and encryption methods are specified.

- Identity is at the transport level, and not at the MIH level.

- The MIH payload is opaque to the transport.

- MoS in the same domain may have implicit trust between each other.

- MoS in the home domain of the MN are implicitly trusted by the MN.

- All communication is at L3 or above.

- The network architecture of the MoS is not defined.

Also in the IETF MIPSHOP WG, [11] describes the problem space for the IETF L3 or above transport solution including the following key points on security:

- The transport protocol and security are not specific to a link type.

- From section 7 Security Requirements:

  - The IS PoS may require authorization of the client in some applications, not in others.

  - With the ES and CS the MN and PoS require mutual authentication.

  - MIH protocol confidentiality and integrity is needed in some scenarios.

  - An implementation should be possible in some scenarios that doesn’t require additional identities beyond those needed for network access.

  - DoS attacks on the network PoS from clients of each service should be evaluated.

- From section 10.4 IETF security requirements:

  - Mutual authentication between two MIH nodes must be possible.

  - One way authentication between two MIH nodes may be possible.

  - Integrity protection must be available.

  - Confidentiality may be available.

  - Replay attack protection must be available.

  - DoS protection may be available.

  - The transport security does not depend on the MIH protocol.

The above descriptions of the MIH services security space are from the context of designing a transport protocol for the MIH protocol. It is useful to consider that these requirements are fulfilled by the higher layer transport. As a result the security requirements for the services themselves, and for use and transport of the MIH protocol at L2, may be different.

In the IEEE 802.11 document [14] the problem space for use of MIH services over 802.11 networks is bounded by these key points:

- In section 11.10.4, MIH services can be discovered in state-1 (unauthenticated state.)

- The IS service can operate in state-1.

- The ES and CS services may operate only in state-3 (authenticated, link security established). It is implied that ES and CS will be transported in IP packets.

In the IEEE 802.16 document [15]  the problem space for use of MIH services over 802.16 networks is similarly bounded by these key points:

- In 6.3.2.3.9, 11.8.10 and table 358 (see MIH Capability Support row), MIH services can be discovered before authorization (also called “during network entry”) using the MIH Initial Service Request.

- In section 6.3.9.8 and 6.3.25 the BS and MS may perform IS protocol exchange before authorization using Privacy Key Management (PKM) messages.

- MIH services may operate in authenticated state without IP transport using MOB_MIH-MSG messages on the Primary Management connection (see 6.3.2.3.62.)

- In I.5 MIH Exchange procedure, Figure I.11 the caption implies that MIH message exchange before authorization is pre-authenticated.

These specifications illustrate there are some considerations needed for using MIH services capability discovery and IS in unauthenticated state, using L2 transport.

From the IAB, [16] discusses link events or indications, and describes some security concerns. The MIH ES is given as an example of a service providing link event reporting in section 1.3. Key points raised are:

- Transport of link events should not introduce new security vulnerabilities in the related unsecured routing protocols.

- Transported link indications need to be “valid”, so the receiving host can accept them even from off-link senders.

- A receiving host needs a way of mapping the received indication to the relevant higher layer services or users.

The above concerns are primarily from the perspective of how the event service might negatively affect the routing or transport services of the network and transport layers.

None of the existing problem space descriptions discuss the need for perfect forward secrecy.