| Project | IEEE 802.21a <https://mentor.ieee.org/802.21> |
| --- | --- |
| Title | Option I in work item 2 – Protection on MIH messages provided by Transport Protocol |
| DCN | 21-09-00xx-00-0Sec |
| Date Submitted | May 5, 2010 |
| Source(s) | Lily Chen (NIST) |
| Re: | Discuss at IEEE 802.21 session #37, May 10-13, 2010 |
| Abstract | This document discusses option I of work item 2: use the protections provided by the MIH transport protocols. |
| Purpose | MIH messages can be transported by layer 2 or layer 3 protocols such as IEEE 802.11 or IP. These transport protocols can provide certain security protections on transported MIH messages. This document discuss pros and cons for the protections provided through transport protocols. . |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

# 1. Introduction

MIH messages can be carried over wireless protocols in layer 2 such as defined in IEEE 802.11u or layer 3 as defined in IETF RFC 5677. These protocols have certain built-in protections. In the following we will discuss the pros and cons for these protection mechanisms.

# 2. Protection Through IPsec

When MIH messages are transported over IP as defined in RFC 5677, it may be protected by IPsec. When IPv6 is implemented in a mobile node and a PoS, then IPsec is mandatory. In this case, MIH messages are protected at IP layer as an IP payload in each IPsec packet as described in Figure 1.

IP header

IPsec header

MIH Message

IP Payload

Figure 1. MIH message is protected as an IPsec Payload.

The IPsec Security Associations (SAs) are established through Internet Key Exchange (IKEv1 or IKEv2). They are heavy weight protocols. However, once the SAs are established, they are used to protect all the IP packets between a MN and a PoS.

First, IPsec protocols are well defined and can provide proper protections for its IP payload. When it is used in tunnel mode between a MN and a PoS, it can provide end-to-end protections.

Using IPsec will not require any changes to either MIH protocol or IPsec.

However, the protections through IPsec are not MIH specific. For example, the authentication provided by IPsec can only assure that the packet is from an IP node with a given IP address and the data was not modified when it is transmitted. Even though the IP address may be the address of a PoS with which a MN is communicating through MIH protocol, a MIHF in MN can not be assured about the source of the data nor the integrity of the data. In fact, MIHF, either in a MN or a PoS, will not be aware of the protections.

# 3. Protection Through (D) TLS

In RFC 5677, it is assumed that TCP or UDP is used as a transport layer protocol as illustrated in Figure 2. In this case, if a MN and a PoS establish a (D)TLS session, where a MN is a client and a PoS is the server, then MIH messages can be protected through (D)TLS as application data.

MIH User

MIHF

TCP/UDP

Figure 2. RFC 5677 Architecture

Again, (D)TLS are well established protocols. They are practical and efficient. The MIHF identities can be used to identify the client and the server. The protections are end-to-end.

Even though (D)TLS are more efficient than IKE protocol, they are session based protocols and must be executed for each session between a MN and a PoS. However, it can resume a session without executing the whole protocol, when the previous session is not expired.

An important issue is that in order to use (D)TLS to protect MIH messages, it may need to assign a new port to distinguish TLS protected MIH from MIH without protection. In RFC 5677, a new TCP port and a new UDP port have been assigned for MIH protocol. If (D)TLS are used to protect MIH, then it will require a change in RFC 5677.

# 4. Protection Through L2

When MIH messages are transported over a layer 2 protocol, the protections may be provided through the layer 2 protocol such as RSNA in IEEE 802.11.

Obviously, the protection is not MIH specific. It may or may not be end-to-end, depending on whether the PoS is a PoA. In fact, the protections in layer 2 are usually established with L2 identifiers for a MN and a PoS such as MAC address.

Furthermore, if MIH messages are transported over different layer 2 protocols, then the protections are also different.

On the other hand, such protections through a layer 2 protocol will not require any change either on MIH protocol or layer 2 protocol.

# 5. Summary

Among all the above discussed protections through transport protocols, (D)TLS is the closest to MIH specific protections. However, it requires new port assignment.

IPsec is not MIH specific. However, it provides uniform protections on MIH messages. Especially in case of IPv6, IPsec is mandatory.

L2 protections do not require any change to MIH protocol. It may carry less overhead due to its lower layer character. But since different L2 protocols will apply different protections. They may not be uniform in the sense of security strength.

# 6. Conclusion

The protections provided through transport protocols, together with pros and cons for each of them, should be included in 21a specification as an option for the case when MIH specific protections can not be established.