IEEE P802.22
Wireless RANs

| Proposed Text Changes for Distributed SCH Design |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Cheng Shan | Samsung Electronics | S. Korea | +82 31 279 7557 | cheng.shan@samsung.com |
| Sangbum Kim | Samsung Electronics | S. Korea | +82 31 279 7092 | sb07.kim@samsung.com |
| Euntaek Lim | Samsung Electronics | S. Korea | +82 31 279 5917 | et.lim@samsung.com |
| Junhong Hui | Samsung Electronics | China | +86 10 5925 3333 3108 | jh.hui@samsung.com |

Abstract

This document proposes related text changes on distributed SCH design, according to the motion passed in Jan. 2009. Revision was done based on Draft v1.4.

- add a new sub-clause under section 6.4 to address both superframe structures under normal mode and coexistence mode.

- modify Table 1 superframe control header format in sub-clause 6.6.1

- add a management message to support quick frame switch

- revise the CPE initialization procedure description and related figures.

- SCH PHY description.

This contribution can be regarded to address the following comments: 131, 132, 136, 149, 150, 664, and 670, but not limit to those comments.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

I. Related Comments

This contribution is to (but not limit to) address the following comments: 131, 132, 136, 149, 150, 664, and 670.

II. Proposed Text Changes

  - General Superframe Structure

The IEEE 802.22 WRAN system adopts two operational modes: normal mode and coexistence mode. In normal mode, one WRAN cell occupies one TV channel and operates on all the frames in a super-frame; while in coexistence mode, multiple WRAN cells share the same TV channel and each WRAN cell operates on one or several frames exclusively with coexisting cells.

When operating in normal mode, a WRAN cell shall transmit a super-frame control header (SCH) at the first frame of a super-frame on the operating TV channel; when operating in coexistence mode, a WRAN cell shall transmit a super-frame control header at the first allocated operational frame of a super-frame on the operating TV channel. The structure of SCH for both normal mode and coexistence mode can be found in clause 6.6.1.

    - General Superframe Structure for Normal Mode

The superframe structure employed in the MAC is depicted in Figure 13, where it can be seen that it is comprised of a first frame constituted of the following:

- a PHY superframe preamble – see clause Error: Reference source not found

- a PHY frame preamble – see clause Error: Reference source not found

- a SCH – see clause Error: Reference source not found, and

- the rest of the first frame including its frame header and data payload.

This first frame is then followed by 15 frames which each include a frame preamble, frame header and the data payload – see clause Error: Reference source not found.

At the beginning of every superframe, the BS shall transmit the superframe preamble and SCH (with a known modulation/coding) on the operating TV channel. In order to synchronize with a base station, a CPE must receive the SCH to establish communication with the BS. During the lifetime of a superframe, 16 MAC frames are transmitted. During each MAC frame, the BS has the responsibility to manage the upstream and downstream operations, which may include ordinary data communication, measurement activities, coexistence procedures, and so on.

The superframe shall start with a superframe preamble, followed by the first frame preamble, the superframe control header (SCH) and finally the first frame payload. The first frame payload shall be reduced by two symbols to keep the frame size consistent.

- General superframe structure

    - General Superframe Structure for Coexistence Mode

The superframe structure in coexistence mode is shown in Figure 14. The coexistence mode is for the scenario when multiple BSs have to share the same TV channel, however, frequency reuse factor cannot be maintain as one due to their mutual interference. In this case, these BSs shall share the TV channel by per frame based access, i.e., each BS uses one or several frames exclusively with its neighboring BSs in a superframe. The negotiation process of frame allocation can be found in section xxx.yyy.

In coexistence mode, the BS transmits its superframe preamble, frame preamble and SCH on the operating TV channel during the first allocated active frame. The BS and CPEs in a WRAN cell only transmit during the active frames allocated to the WRAN cell. The BS and CPEs in a WRAN cell shall not perform transmission during the non-active frames except the SCWs therein, if scheduled. During the non-active frames, the BS and CPEs in the WRAN cell can monitor the TV channel for any transmission from neighbouring WRAN cells to improve coexistence.

Figure 14	General superframe structure in Coexistence mode

    - Superframe Control Header

The SCH specification is shown in Table 282. Since the SCH decoding is critical, the SCH shall be transmitted using the modulation described in section Error: Reference source not found. The SCH provides information about the 802.22 cell, which is used to support coexistence with incumbents, support self-coexistence mechanisms, and support the intra-frame and inter-frame mechanisms for management of quiet periods for sensing as described in Error: Reference source not found.

Transmission of a SCH indicates that the WRAN cell is operating under one of the two possible coexistence modes: normal mode, or coexistence mode. In normal mode, the WRAN cell uses all the sixteen frames in a superframe for its communication, while in coexistence mode, the WRAN cell uses one or several frames in a superframe for its communication. In normal mode, the SCH is transmitted during the first frame of a superframe; in coexistence mode, the SCH is transmitted during the first assigned frame in a superframe. The general frame structure is illustrated in section 6.4.

Since a SCH may contain further IEs, the Length field is used to specify the total length of the SCH.

It is understood that all bits indicated as reserved in Tables throughout this standard shall be transmitted as zeros unless otherwise explicitly specified.

- — Superframe control header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Superframe_Control_Header_Format() { |  | 1 OFDM symbol long and transmitted with modulation/coding described in section Error: Reference source not found |
| Superframe Mode | 1 bit | Normal Mode = 0 Coexistence Mode = 1 |
| If Superframe Mode=1 { |  |  |
| SCH_Offset | 4 bits | Indicate the frame index of current frame in a superframe. |
| Frame Allocaiton MAP | 16 bits | Indicate which frames in the present superframe are allocated to the present WRAN cell. |
| } |  |  |
| Superframe Number | 8 bits | Positive integer that represents the superframe number (modulo 255). This field shall be incremented by 1 upon every new superframe. |
| TTQP | 12 bits | Time To Quiet Period Used for in-band inter-frame sensing, it indicates the time span between the transmission of this information and the next scheduled quiet period for inter-frame sensing. The 8 left most bits index the superframe number and the 4 right most bits index the frame number when the next scheduled quiet period for inter-frame sensing will start. |
| DQP | 4 bits | Duration of Quiet Period Used for in-band inter-frame sensing, it indicates the duration of the next scheduled quiet period. If this field is set to a value different from 0 (zero), it indicates the number of frames starting from TTQP that shall be used to perform inter-frame sensing. If this field is set to 0, it cancels the next scheduled quiet period for inter-frame sensing or indicates that no inter-frame sensing are currently scheduled] |
| Tx ID | 48 bits | MAC address that uniquely identifies the CPE or BS transmitting the SCH. |
| CN | 8 bits | Channel Number Indicates the physical TV channel number used by the BS. |
| CP | 2 bit | Cyclic Prefix Factor Specifies the size of the cyclic prefix used by the PHY in the frame transmissions in this superframe. Pre-determined values are: 1 = ¼ symbol period 2= 1/8 symbol period 3= 1/16 symbol period 4= 1/32 symbol period |
| Length | 8 bits | The length in bytes of the information following the SCH. |
| Intra-frame Sensing Cycle Length | 8 bits | Specified in number of superframes, it indicates the number of superfames for which the intra-frame sensing specification is valid. If this field is set to 0, no intra-frame sensing is scheduled or the current intra-frame sensing is canceled. |
| Intra-frame Sensing Cycle Offset | 8 bits | Valid only if intra-frame Sensing Cycle Length > 0. Used for in-band intra-frame sensing. Specified in number of superframes, it indicates the offset from this SCH transmission to the beginning of the first superframe in the current intra-frame sensing cycle. |
| Intra-frame Sensing Cycle Frame Bitmap | 16 bits | Valid only if intra-frame Sensing Cycle Length > 0. Valid for a unit of superframe, each bit in the bitmap corresponds to one frame within the superframe. If the bit is set to 0, intra-frame sensing shall not be performed in the corresponding frame. If the bit is set to 1, intra-frame sensing shall be performed within the frame for the duration specified by Intra-frame Sensing Duration. This bitmap applies to all superframes within the Intra-frame Sensing Cycle Length. |
| Intra-frame Sensing Duration | 5 bits | Valid only if Intra-frame Sensing Cycle Length > 0. If this field is set to a value different from 0 (zero): It indicates the number of symbols starting from the end of the frame that shall be used to perform intra-frame sensing. |
| IEs | Variable (integer of bytes) | Optional Information Elements which could be transmitted with the SCH. They are: MAC version (Error: Reference source not found) Current transmit power (Error: Reference source not found) Location configuration information (Error: Reference source not found) DTV acknowledgement (Error: Reference source not found) Wireless microphone acknowledgement (Error: Reference source not found) |
| HCS | 8 bits | Header Check Sequence See Error: Reference source not found |
| Reserved | 4 bits | All bits shall be set to zero. |
| } |  |  |

(Insert a new management message FRM_UPD)

  - Management Messages

…

- — Management messages

| Type | Message | Description | Reference | Class of connection |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| 56 | FSL-REQ | Frame Slide Request | Error: Reference source not found | Basic, Multicast Management or Broadcast |
| 57 | FSL-RSP | Frame Slide Response | Error: Reference source not found | Basic |
| 58 | FRM_UPD | The first active frame allocation update in coexistence mode | 6.9.29 | Basic |

    - Active Frame Allocation Management

In coexistence mode, multiple WRAN cells share the same TV channel by per frame-based access. The active frame allocation may change from one superframe to another. When the location of the first active frame is changed in next superframe, the BS shall send a first active frame allocation update information within the present superframe to indicate all its associated CPEs.

| Syntax | Size | Notes |
| --- | --- | --- |
| FRM-UPD_Message_Format() { |  |  |
| Management Message Type = 58 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Frame Index of the first active frame in next superframe | 4 bits | Indicate the frame index (0-15) of the first active frame allocated in next superframe. |
| Reserved | 4 bits |  |
| } |  |  |

    - CPE Initialization

…

First and foremost, the MAC does not presuppose any pre-assigned channel where a CPE is able to look for a BS given the time-varying and unpredictable nature of channel occupancy. Hence, the first task a CPE must perform in attempting to join a network is to scan the set of channels for BSs and incumbent services on which the transmissions of the CPE might interfere. Since the BS shall send concatenated OFDM symbols of a superframe preamble, a frame preamble and an SCH once every superframe in its operating channel (see 6.4, Error: Reference source not found), the CPE will recognize the existence of a BS transmission and, if appropriate, proceed with the CPE initialization procedure with the corresponding BS.

…

- Obtaining downstream parameters

(Insert a new sub-clause after section 6.21.2.1.3.2.3)

            - Discovery during non-active frames in coexistence mode

In coexistence mode, both BS and associated CPEs in the WRAN cell can listen to the medium for network discovery during non-active frames. The discovery can be done by either searching for a coexistence beacon or searching for a SCH from neighbor cells.

  - Superframe and Frame Structures

The basic superframe structure and frame structure are shown in Figure 13-15. See Error: Reference source not found and Error: Reference source not found for a full description of the superframe and frame structures.

The superframe shall consist of 16 frames of 10ms each. Each frame contains a preamble, header, and data bursts.

For both normal and coexistence operational modes, in the first allocated active frame, the first symbol shall be the superframe preamble, followed by a frame preamble symbol. The third symbol shall be the SCH, and the fourth symbol shall contain the FCH and, when needed, DS-MAP, US-MAP, DCD and UCD. The SCH is transmitted to support coexistence with incumbents through the scheduling of the quiet periods, self-coexistence, and so on. The FCH specifies the length of the four (DS-MAP, US-MAP, DCD, UCD) downstream messages that may immediately follow the FCH. The first active frame payload will contain two fewer symbols than normal frames to keep the length to 10 ms.

The other active frames of the superframe contain a frame preamble, the FCH and the DS-MAP, US-MAP, DCD, and UCD messages, and the data bursts.