IEEE P802.22
Wireless RANs

| Suggested Text Changes for 22b Segmentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-11-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Seung-Keun Park | ETRI | Korea | +82-42-860-5993 | seungkp@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |
| Ranga Reddy | Self | US |  | ranga.reddy@me.com |

Abstract

This is the suggested text changes for 802.22b segmentation scheme. It is the resolution for 1st WG Letter Ballot CID #218 and 2nd WG Letter Ballot CID #45.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related Comment

- This is the resolution for 1st WG Letter Ballot CID # 218 and 2nd WG Letter Ballot CID #45 as described in the following table.

- 2. Proposed Resolution

- In the September 2014 F2F meeting, we have reached on the consensus regarding segmentation scheme. The key features are summaried in the document 22-14/0100r2.

- 3. Proposed Text Changes

- Change the section 7.4b.2 as follows:

7.4b.2 General frame structure for a distributed scheduling mode

Each of the downstream and upstream subframes for a distributed scheduling mode may include two zones: access zone (AZ) and distributed relay zone (DRZ) as shown in Figure E1. Each AZ in the downstream and upstream subframes is used for transmission between an A-BS and CPEs (distributed scheduling A-CPEs or S-CPEs), while each DRZ in the downstream and upstream subframe is used for transmission between a distributed scheduling A-CPE and S-CPEs.

The both of AZs and DRZs in the downstream and upstream subframes are scheduled by an A-BS. For a distributed scheduling mode, the AZs in the downstream and upstream subframes are managed by an A-BS, while the DRZs in the downstream and upstream subframes are controlled by a distributed scheduling A-CPE, which is capable of configuring and maintaining a local cell within an A-WRAN cell. The AZ of the downstream subframe shall appear earlier than other zones of the AZ of the upstream subframe or the DRZs of the downstream and upstream subframes. Except for the AZ of the downstream, the order of the AZ of the upstream subframe, the DRZ of the downstream subframe, and the DRZ of the upstream subframe in the frame may be changed during the operations.

For the A-WRAN on either PHY mode, the subchannels of the DRZs in the downstream and upstream subframes can be grouped by 3 segments with the fixed number of subchannels as shown in Figure 00. The segment bandwidth is scalable up to occupied channel bandwidth. For PHY mode 1 and PHY mode 2, each segment includes the number of subchannels defined in Table 00. For the DS of PHY mode 2, the number of tiles per occupied channel bandwidth is not divided by 3. Thus the segment 1, 2, and 3 has the different number of tiles, i.e., 72, 68, and 68 tiles, respectively. Figure 01 shows all the combinations of segment bandwidth and start position. There are three different bandwidth of segment and three different start position of segment. As shown in Figure 01, the number of segment bandwidth mode is 3, 2, and 1 for the 1st, 2nd, and 3rd start position, respectively. The configuration of segmentation scheme can be identified by local frame preamble as defined in 9.4.1.5 and 9a.4.1.3. The segmentation can be scheduled by the A-BS, and each segment is assigned to the different distributed scheduling A-CPEs. This segmentation is used to increase network capacity.

- Figure 00-Segment structure

- Table 00-Segment bandwidth

| - PHY mode - Segment index | - PHY mode 1 |  | - PHY mode 2 |  |
| --- | --- | --- | --- | --- |
|  | - DS - (subchannels) | - US - (subchannels) | - DS - (tiles) | - US - (tiles) |
| - Size of Segment 1 | - 10 | - 20 | - 72 | - 70 |
| - Size of Segment 2 | - 10 | - 20 | - 68 | - 70 |
| - Size of Segment 3 | - 10 | - 20 | - 68 | - 70 |

- Figure 01-Combination of start position of segment and segment bandwidth

- Change the section 7.7.2.1.2.2 as follows:

7.7.2.1.2.2 DS Multi-Zone Configuration IE

A CPE shall be able to decode the DS Multi-Zone Configuration IE shown in Table G1. An A-BS shall transmit

- this IE for multi-hop relay operations.

- Table G1—DS Multi-Zone Configuration IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| - DS Multi-Zone Configuration_IE() { |  |  |
| - Multi-Zone Configuration{ |  |  |
| - Number of zones | - 8 bits | Number of zones including access and relay zones. Number of zones (0) is not available of DS. Number of zone (1) shall be access zone. |
| - For(i=1; i =< Number of zones; i++){ |  |  |
| - Zone Index | - 8 bits | - Increase the index from 0 to Number of Zones-1 |
| - Zone Mode | - 2 bits | 0: access zone 1: centralized relay zone - 2: distributed relay zone |
| - Start Position of Segment | - 2 bits | - 00: 0 - 01: 1 - 10: 2 - 11: If ‘Zone Mode’ is set to Access or Centralized Relay Zone |
| - Segment Bandwidth | - 2 bits | 00: one segment from start position 01: two segments from start position 10: three segments from start position 11: If ‘Zone Mode’ is set to Access or Centralized Relay Zone |
| - PHY Mode | - 1 bit | 0: PHY Mode 1 1: PHY Mode 2 |
| - } |  |  |
| - if (Zone mode == 2) { |  | - Distributed Relay Zone (DRZ) mode |
| - SID | - 13 bits | - SID of distributed scheduling A-CPE |
| - } |  |  |
| - } |  |  |
| - } |  |  |

- Insert the section 9.4.1.5 as follows:

- 9.4.1.5 Local frame preamble

The local frame preamble shall use the TCP=1/4TFFT. Three different preamble carrier sets are defined, differing in the allocation of subcarriers. Those subcarriers are modulated using a boosted BPSK modulation with a specific pseudo-noise (PN) code.

The preamble carrier-sets are defined using Equation (xx1).

- PreambleCarrierSetn = n + 3k                                                                                 (xx1)

where

PreambleCarrierSetn specifies all subcarriers allocated to the specific preamble

n is the designating number of the preamble carrier-set indexed 0, 1, and 2

k is a running index. 0...559

Each start position of segment uses a preamble composed of a single carrier-set in the following manner:

— Start position of segment 0 uses preamble carrier-set 0 (n =0).

— Start position of segment 1 uses preamble carrier-set 1 (n =1).

— Start position of segment 2 uses preamble carrier-set 2 (n =2).

In the case of start position of segment 0, the DC carrier will not be modulated at all, and the appropriate PN will be discarded. Therefore, the DC carrier shall always be zeroed.

Each start position of segment eventually modulates each third subcarrier. As an example, Figure XX1 depicts the preamble of start position of segment 0. In this figure, subcarrier 0 corresponds to the first subcarrier used in the preamble symbol.

Figure XX1—Example of basic structure of preamble (for the case of n =0)

The PN series modulating the preamble carrier-set is defined in Table XX1. The series modulated depends on the start position of segment and segment bandwidth used. The defined series shall be mapped onto the preamble subcarriers in ascending order. Table XX1 includes the PN sequence in a hexadecimal format. The value of the PN is obtained by converting the series to a binary series (Wk) and mapping the PN starting from the MSB of each symbol to the LSB (0 mapped to +1 and 1 mapped to –1). For example, for Local Frame Preamble 0 (the first row of Table XX1), Wk = 001001010001..., and the mapping shall follow: –1 –1 +1 –1 –1 +1 –1 +1 –1 –1 –1 +1....

For the preamble symbol, there will be 184 guard band subcarriers on each side of the spectrum.

The symbols in the DS preamble shall be modulated according to Equation (xx2).

- RePreambleModulated=2⋅3⋅12−Wk, ImPreambleModulated=0          (xx2)

- Table XX1—Local frame preamble modulation series for segmentation scheme

| - Preamble Type | - Start Position of Segment | - Segment Bandwidth | - Series to modulate (Wk) |
| --- | --- | --- | --- |
| - Local Frame Preamble 0 | - 0 | - 1 segment | - 0x251D994101EDA04D8BD0B8EA6FA20AE590C2CC199AB083C6AE61F091F2DD41D989EC164B1481D611BE9CEA0094AFE9DB56A4763F55B26E54EAB73ACD7D4BBA64C1421BC3EB9D |
| - Local Frame Preamble 1 | - 0 | - 2 segments | - 0x572B008CAE935937061963E9567C204CB881C66F6C70DC9316A4006F9CDF449C19E5EC29CCFC42786A82330FC7279F99F1DFCC4246B1234F792B623341EBA8DBDAC7FD337206 |
| - Local Frame Preamble 2 | - 0 | - 3 segments | - 0x9D7B838B01289DA478424A99E6B5C35E7D6D79FCC9ADEFEE741BFBD48261B9A427AE8994EB230CF27D770B7CDD53A01821C63ADD01236D481BFEF0085FF21DF2044E054E9113 |
| - Local Frame Preamble 3 | - 1 | - 1 segments | - 0xBBF8D4EE09564B33B49FFA37179626165084481424D61EC7592A97D80A66FA6797B1A0C493987566D49A6B2786BC9706F8168264BD24F9A8BD86EEBC0211848DFB2B477E2CF3 |
| - Local Frame Preamble 4 | - 1 | - 2 segments | - 0x3A13D1C743143CE5E66B0AC149147AA2E4624EF574EB4D49FC698483BFEB60FB07B286EDB255BC19C0AD0D53669AF9E41E7CF22795EAF0CDCB9F1EBAF6979B05A0373A3C7403 |
| - Local Frame Preamble 5 | - 2 | - 1 segments | - 0x9D6E088D4BF521B816766CDD6DEC121DD3C2101F30B8479CADA2EDA38CC97B865D78BDDC1A50843DCC6547D9B4CE4EF1A3E9661C42648013DB35E2E69D7404D52DACA4886F5D |

- Insert the section 9.4.1.6 as follows:

- 9.4.1.6 Local CBP preamble

The local CBP preamble shall have a duration of 1 OFDM symbol. The preamble structure is used by the same nanner as described in 9.4.1.5. The PN series modulating the local CBP preamble carrier-set is defined in Table XX2. The series modulated depends on the start position of segment and segment bandwidth used. The defined series shall be mapped onto the local CBP preamble subcarriers in ascending order. Table XX2 includes the PN sequence in a hexadecimal format. The value of the PN is obtained by the same manner as described in 9.4.1.5.

- Table XX2—Local CBP preamble modulation series

| - Preamble Type | - Start Position of Segment | - Segment Bandwidth | - Series to modulate (Wk) |
| --- | --- | --- | --- |
| - Local CBP Preamble 0 | - 0 | - 1 segment | - 0xF3E5596544A8D30FF9DAA3AB6CBE5EC769CAB9F0E2F9FF3D1D27619481C8A1A3EC4286F8207F1C69816CC13E995C38C1E4D62AD594B2B02CDEE05A0F1BD18BCA09FC2B1DC72B |
| - Local CBP Preamble 1 | - 0 | - 2 segments | - 0xFCAEC6B1C7027793A48849AD347AE875408955E7F2AAC5FF9904A2D0D904C931795A1BDA94D9DCABA8BA77152A69B89F008680F34505343E591F3EB3CF103BF97DA73BC87FC9 |
| - Local CBP Preamble 2 | - 0 | - 3 segments | - 0x1F6C5C088F9FF6E85171FF5434B95DEB270F1C926F738B8167566D24EC3F7A1BEDC7CC3B913EC2DB60B4103739C07C10DB5D036A9B48223CE4037E148205DD8177084A97E739 |
| - Local CBP Preamble 3 | - 1 | - 1 segment | - 0x1A6988580366EDA426EDA5EC69AEC0B9D77D733AC09338B2E66BCBA5F1960FEE2C08477CB6F80DB152B4EF01E7FBFD5961A16C954EFEB629124F9789B34411713ED05A0EF02C |
| - Local CBP Preamble 4 | - 1 | - 2 segments | - 0xFA54537894CC41ADE3960B29C89DCAC398CEDFB2AB6BCB61D02B877208A71DD8760FFCABA593996852CAE9AE7E7FF7633EAF24CD2AAC1229E10D9847837CC20C46270E8F824F |
| - Local CBP Preamble 5 | - 2 | - 1 segment | - 0x8C88EE32C92B5741BBF6A7026E0C268520653A0A948EA318A316508A4602A75DED6AF1D33BCB0B83E8A3C7372E0779FC5C10AA16063AF909031455A18EA324FBD5FECF499D83 |

- Change the section 9a.4.1.1 as follows:

9a.4.1.1 Frame preamble

The first symbol of the DS transmission is the preamble. Three different preamble carrier sets are defined, differing in the allocation of subcarriers. Those subcarriers are modulated using a boosted BPSK modulation with a specific pseudo-noise (PN) code.

The preamble carrier-sets are defined using Equation (3).

- PreambleCarrierSetn = n + 3k                                                                                 (3)

where

PreambleCarrierSetn specifies all subcarriers allocated to the specific preamble

n is the designating number of the preamble carrier-set indexed 0, 1, and 2

k is a running index. 0...283

Each start position of segment uses a preamble composed of a single carrier-set in the following manner:

— Start position of segment 0 uses preamble carrier-set 0 (n =0).

— Start position of segment 1 uses preamble carrier-set 1 (n =1).

— Start position of segment 2 uses preamble carrier-set 2 (n =2).

In the case of start position of segment 0, the DC carrier will not be modulated at all, and the appropriate PN will be discarded. Therefore, the DC carrier shall always be zeroed.

Each start position of segment eventually modulates each third subcarrier. As an example, Figure EL1 depicts the preamble of start position of segment 0. In this figure, subcarrier 0 corresponds to the first subcarrier used in the preamble symbol.

Figure EL1—Example of basic structure of preamble (for the case of n =0)

The PN series modulating the preamble carrier-set is defined in Table HW1. The series modulated depends on the start position of segment and segment bandwidth used. The defined series shall be mapped onto the preamble subcarriers in ascending order. Table HW1 includes the PN sequence in a hexadecimal format. The value of the PN is obtained by converting the series to a binary series (Wk) and mapping the PN starting from the MSB of each symbol to the LSB (0 mapped to +1 and 1 mapped to –1). For example, for Frame Preamble (the first row of Table HW1), Wk = 101001101111..., and the mapping shall follow: +1 –1 +1 –1 –1 +1 +1 –1 +1 +1 +1 +1....

For the preamble symbol, there will be 86 guard band subcarriers on each side of the spectrum.

The symbols in the DS preamble shall be modulated according to Equation (4).

- RePreambleModulated=2⋅3⋅12−Wk, ImPreambleModulated=0                  (4)

- Table HW1—Frame preamble and local frame preamble modulation series

| - Preamble Type | - Start Position of Segment | - Segment Bandwidth | - Series to modulate (Wk) |
| --- | --- | --- | --- |
| - Frame Preamble | - - | - - | - 0xA6F294537B285E1844677D133E4D53CCB1F182DE00489E53E6B6E77065C7EE7D0ADBEAF |
| - Local Frame Preamble 0 | - 0 | - 1 segment | - 0x668321CBBE7F462E6C2A07E8BBDA2C7F7946D5F69E35AC8ACF7D64AB4A33C467001F3B2 |
| - Local Frame Preamble 1 | - 0 | - 2 segments | - 0x1C75D30B2DF72CEC9117A0BD8EAF8E0502461FC07456AC906ADE03E9B5AB5E1D3F98C6E |
| - Local Frame Preamble 2 | - 0 | - 3 segments | - 0x5F9A2E5CA7CC69A5227104FB1CC2262809F3B10D0542B9BDFDA4A73A7046096DF0E8D3D |
| - Local Frame Preamble 3 | - 1 | - 1 segments | - 0x82F8A0AB918138D84BB86224F6C342D81BC8BFE791CA9EB54096159D672E91C6E13032F |
| - Local Frame Preamble 4 | - 1 | - 2 segments | - 0xEE27E59B84CCF15BB1565EF90D478CD2C49EE8A70DE368EED7C9420B0C6FFAF9AF035FC |
| - Local Frame Preamble 5 | - 2 | - 1 segments | - 0xC1DF5AE28D1CA6A8917BCDAF4E73BD93F931C44F93C3F12F0132FB643EFD5885C8B2BC |

- Change the section 9a.4.1.2 as follows:

9a.4.1.2 CBP preamble

The CBP and local CBP preamble shall have a duration of 1 OFDM symbol. The preamble structure is used by the same nanner as described in 9a.4.1.1. The preamble carrier-set 0 is used for CBP preamble. The PN series modulating the CBP and local CBP preamble carrier-set is defined in Table HX1. The series modulated depends on the start position of segment and segment bandwidth used. The defined series shall be mapped onto the CBP and local CBP preamble subcarriers in ascending order. Table HX1 includes the PN sequence in a hexadecimal format. The value of the PN is obtained by the same manner as described in 9a.4.1.1.

- Table HX1—CBP preamble and local CBP preamble modulation series

| - Preamble Type | - Start Position of Segment | - Segment Bandwidth | - Series to modulate (Wk) |
| --- | --- | --- | --- |
| - CBP Preamble | - - | - - | - 0x61AF26BD39A9FFF52826625E04ADA299385A373FA946D837D754E6CFEBB26F5C03B87CF |
| - Local CBP Preamble 0 | - 0 | - 1 segment | - 0xD77D97CDB93DBEAA65CAFA146F40D72B5E80944F750E07325DC164ED60F32434BC7187D |
| - Local CBP Preamble 1 | - 0 | - 2 segments | - 0x4529D9CA65AF49C1C39BDC18CFAB87E03FE4DAFC0A48FF1457D46B0DF66B414A23ACDDB |
| - Local CBP Preamble 2 | - 0 | - 3 segments | - 0x33AC0261DAA57C1D611EBA1C730D50AFEE5BE3E849030A4E891BC8C5F4C78DCDDFEA263 |
| - Local CBP Preamble 3 | - 1 | - 1 segments | - 0xBED48C704F02A84F03BCD299D919DA56F7B71EDF8A0F8A25E8F8496F95A44CE2B9F74C9 |
| - Local CBP Preamble 4 | - 1 | - 2 segments | - 0x0ECCBE0902EBF4B4C29506014A3706622784B7B2D5153E10AD3112DC5E45277A32E79DE |
| - Local CBP Preamble 5 | - 2 | - 1 segments | - 0x7CB4937889C7DFD9AA2D37235E06F993D3D4F5D515B39CA652F62397C08457D66BC5A36 |